| # Central Fab Group - Compound Semiconductor | Safety Regulations for Compound Semiconductor Laboratories. |
| --- | --- |
| - Approved eye protection must be worn at all times when working in Compound Semiconductor Laboratories. - A full face visor must be worn when working at the chemical fumehoods. - Hotplates must be attended while on. - No wet chemical processing to be done solo outside normal hours. - No chemicals are to be left in beakers overnight. - Last person leaving the laboratory is responsible for leaving the laboratory in a safe state. (See checklists at the doors – TYNDALL-035F4). - Gloves and respiratory protection must be used when handling GaAs dust. - New work with hazardous chemicals and other clearly hazardous experiments to be cleared with the Manager of the Compound Semiconductor Fabrication Laboratory before chemicals or equipment are requisitioned. - All flammable and toxic gases to be kept in extracted gas cabinets. - All new hazardous chemicals to be cleared with Lab manager before they can be used. - Breaking of toxic substrates, i.e. GaAs, to be done in designated filtered extract cabinet only. - Disposal: a) solvents in disposal cans b) semiconductor materials in the III-V waste container c) waste oil in waste oil drums d) electroplating solutions in waste containers e) acids and alkalis diluted 1000X through sink. - Spill Response Actions: a) warn senior personnel within the laboratory and notify the lab manager b) evacuate laboratory and prevent re-entry (if necessary) c) call emergency response group (if necessary) - High Voltage (V>400 Volts). All exposed high voltage components must be inside an interlocked enclosure. |  |